PCB making guideline

Use Fritzing to design PCB – use extra thick wire

(make folder first) Export. svg file

Select two files;

- FILENAME_etch_copper_top_mirror.svg

- FILENAME_etch_mask_top_mirror.svg

Open first file; FILENAME_etch_copper_top_mirror.svg

- Ungroup all parts (command + shift + g)

- Remove nothing but lines and circles

- Fill in every circle in black colour and remove outline

- Resize every circle in 2mm diameter 
(1.473 –> 135%, 1.346 –> 150%)

- Divide image file in few layers not to over 15-minute max.

Open second file; FILENAME_etch_mask_top_mirror.svg

- Ungroup all parts

- Remove nothing but circles

- Remove outline

- Resize every circle in 1mm diameter
(1.473 -> 68%, 1.346 -> 75%)

- Add one additional layer to make PCB outline.
make square - same size as PCB – with empty inside.

- Divide image file in few layers not to over 15-minute max.